# vRAM Booster

Technical Analysis Report

Architecture Review, Beacon Removal & Strategic Direction

Date: 2026-03-25

Repository: DONG-WEI-YANG/vRAM

Commits: d7ac1b2 → fde7877 → e4ee3b9

# 1. Executive Summary

This report documents a comprehensive architecture review of the vRAM Booster project, covering the discovery of critical issues, their resolution, and a strategic pivot away from the OS beacon mechanism toward honest system integration.

## 1.1 Key Findings

- The 99GB commit limit observed in Task Manager is entirely from Windows' auto-managed pagefile (31GB RAM + 68GB C:\pagefile.sys), not from our beacon mechanism.

- The OS beacon (NtCreatePagingFile on C:\) was cosmetic — it inflated commit limit numbers but actual page I/O went to C:\ SSD, not to the external device.

- Seven critical architectural issues were discovered and fixed: thread races, stale handles, multi-instance conflicts, and resource leaks causing .exe file locking.

- The beacon mechanism has been fully removed (−303 lines). The project now reports honest numbers based on actual device connectivity.

# 2. System State Analysis

The following measurements were taken on the development machine to establish ground truth:

| Metric | Value | Source |
| --- | --- | --- |
| Physical RAM | 31.1 GB | Hardware |
| C:\ pagefile.sys | 68 GB (auto) | Windows auto-managed |
| Commit Limit | 99.1 GB | RAM + pagefile |
| Pagefile actual usage | 450 MB (0.6%) | Win32_PageFileUsage |
| Registry PagingFiles | ?:\pagefile.sys | Default (OS-managed) |
| Our beacon (C:\vram_boost.sys) | 1.7 GB (residual) | NtCreatePagingFile |
| External device (E:\) | 500 GB, Removable | VRAM_SD label |

Critical finding: The registry value '?:\pagefile.sys' is Windows' default magic path meaning 'OS chooses disk and size automatically.' This was never modified by our program. The 99GB commit limit is entirely Windows' own configuration.

## 2.1 Residual Files Discovered

Two files created by earlier testing were found on C:\ and could not be deleted at runtime because Windows still held pagefile locks:

- C:\vram_boost.sys (1.7 GB) — created 2026/03/24 by NtCreatePagingFile beacon

- C:\vram_test_pagefile.sys (512 MB) — created 2026/03/24 during development testing

These files require a system reboot to fully remove.

# 3. Issues Discovered & Fixed

## 3.1 Beacon Disconnect Problem (Commit d7ac1b2)

When external devices were removed, the OS beacon pagefile on C:\ remained unchanged, causing Task Manager to continue showing inflated memory numbers. Root cause: MmapSwapEngine's on_state_change callback was never wired to RealBoostEngine.

Fix: Connected the callback chain: MmapSwapEngine._on_disconnect() → on_state_change('degraded') → RealBoostEngine._on_device_state_changed() → recalculate and attempt beacon resize.

## 3.2 EXE File Locking (Commit d7ac1b2)

The built .exe was frequently locked by Windows after closing, preventing updates. Six root causes were identified:

| Root Cause | Location | Severity | Fix |
| --- | --- | --- | --- |
| Thread join timeout 5s (SD I/O can block 30s+) | real_boost.py, mmap_engine.py | Critical | 5s → 15s |
| ThreadPoolExecutor shutdown(wait=False) | striped_swap.py:295 | Critical | wait=True |
| Log FileHandler never closed | hotplug_launcher.py:785 | High | Close in _quit() |
| mmap close failures silently swallowed | mmap_engine.py:220 | High | Error counting |
| No atexit handler | hotplug_launcher.py | High | Added atexit |

## 3.3 Architecture Safety Issues (Commit fde7877)

Seven critical issues were identified and fixed in a single commit:

| # | Issue | Risk | Fix |
| --- | --- | --- | --- |
| 1 | No threading lock on shared state in RealBoostEngine | Race condition → crash | Added _state_lock (RLock) |
| 2 | Block state mutation without lock in MmapSwapEngine | TOCTOU → AttributeError | Added _block_lock |
| 3 | id(engine) as dict key reused after GC | Wrong swap byte count | engine.uid (uuid4) |
| 4 | Multi-instance beacon conflict (C:\vram_boost.sys) | Corrupted accounting | PID-based naming |
| 5 | Beacon max = full swap → C:\ disk filled under pressure | System drive exhaustion | Cap to C:\ free 50% |
| 6 | I/O errors not wired to circuit breaker | Silent data loss post-hibernate | io_errors → breaker |
| 7 | No I/O error reset after device reconnection | Permanent degraded state | reset_io_errors() |

## 3.4 Beacon Removal (Commit e4ee3b9)

Decision: Remove the entire beacon mechanism (−303 lines of code). Replaced with _get_system_pagefile_bytes() that honestly reports existing Windows pagefile state.

Rationale: The beacon created a fake pagefile on C:\ to inflate Task Manager numbers. Actual page I/O never went to the external device. With Windows auto-pagefile already providing 68GB, the beacon added no real value and created cleanup/lifecycle complexity.

# 4. Industry Comparison: Phison SSD VRAM vs OS Swap

A comparison of memory expansion approaches reveals that the basic swap/pagefile mechanism has near-zero technical barrier. The real differentiation occurs at the firmware and protocol layers.

| Layer | Technology | Barrier | Latency | Example |
| --- | --- | --- | --- | --- |
| Application | OS swap / pagefile | None | 100–500 µs | Windows auto PF |
| Driver | Custom block device | Medium | 50–200 µs | zram / zswap |
| Firmware | PCIe BAR mapping | High | 1–50 µs | Phison (possible) |
| Protocol | CXL Type 2/3 | Very High | 0.2–10 µs | CXL memory expander |

Conclusion: If Phison's 'SSD VRAM' only operates at the application layer (Linux swap), it is functionally equivalent to mkswap + swapon — three shell commands with zero technical barrier. The differentiation, if any, lies in firmware-level DRAM cache exposure via PCIe BAR, which requires SSD controller modification.

## 4.1 Traditional Swap Partition vs Our mmap Approach

Traditional swap partitions, swap files, and Windows pagefiles all share the same kernel I/O path: page fault → OS memory manager → block I/O → storage controller → NAND. Performance differences between partition-based and file-based swap are under 1% on modern SSDs. Our mmap approach operates in userspace and does not participate in the OS's global memory management.

# 5. Strategic Direction

Two viable paths forward have been identified, ordered by feasibility:

## 5.1 Path A: Smart Windows Pagefile on External Devices

Concept: Detect when C:\ space is insufficient for adequate pagefile, then create a real Windows pagefile on the external device.

Technical landscape:

| Device Type | Windows Pagefile Allowed? | Our Challenge |
| --- | --- | --- |
| Fixed USB SSD (BusType=USB, not removable) | Yes — Windows allows | None — standard Win32_PageFileSetting WMI |
| SD card (Removable) | No — kernel rejects | High — kernel walks device stack, blocks removable |
| USB flash drive (Removable) | No — kernel rejects | High — same kernel restriction as SD |

For fixed USB SSDs, this is straightforward and provides real system-wide benefit. For removable devices (SD/USB flash), the Windows kernel explicitly blocks pagefile creation by checking the device stack for removable bus types. Bypassing this requires either:

- A custom storage miniport driver that presents the removable device as fixed (medium difficulty, requires driver signing)

- Hooking NtCreatePagingFile's internal validation (high difficulty, fragile across Windows versions)

- Modifying device registry flags to mark removable as fixed (medium difficulty, may cause side effects)

## 5.2 Path B: Low-Speed External Read as Compute Expansion

Concept: Even at low bandwidth (3–50 MB/s), external storage can expand effective compute capacity by serving as a model weight tier. The key insight: AI inference is not bandwidth-bound for the entire model — only the active layers need to be in fast memory at any given time.

Technical approach:

- Layer-aware prefetching: Predict which model layers will be needed next (transformer models have sequential layer access patterns) and prefetch from external storage to RAM before the GPU needs them.

- Double-buffering: While GPU processes layer N from RAM, prefetch layer N+1 from external storage. At 10 MB/s, a 256MB layer takes 25 seconds — but if inference on the current layer takes 30 seconds, the prefetch is hidden.

- Quantization-aware tiering: Keep hot layers (attention heads) in RAM at FP16, store cold layers (FFN) on external storage at INT4/INT8. A 70B model drops from 140GB (FP16) to ~35GB (Q4) — fits on a 64GB SD card.

- Integration with llama.cpp's native mmap: llama.cpp already mmap's model files. Placing GGUF model files directly on the external device and letting llama.cpp mmap them provides transparent OS-level demand paging with zero code changes.

This path has a higher technical barrier but creates genuine differentiation — it cannot be replicated with three shell commands. The value proposition shifts from 'fake bigger numbers' to 'run models that genuinely don't fit in RAM, even on slow storage.'

# 6. Target User Redefinition

The project's value proposition is strongest for resource-constrained devices, not high-end machines:

| User Profile | RAM | C:\ Storage | Our Value |
| --- | --- | --- | --- |
| Budget laptop / mini PC | 8–16 GB | 128 GB eMMC | High |
| Raspberry Pi / SBC | 4–8 GB | 32 GB SD | Very High |
| Mid-range desktop | 16–32 GB | 512 GB SSD | Medium |
| High-end workstation | 32–64 GB | 1–2 TB SSD | Low / None |

The project should be marketed toward users who cannot run desired AI models due to insufficient RAM + pagefile, where external storage provides the missing capacity.

# 7. Commit History

| Hash | Description | Impact |
| --- | --- | --- |
| d7ac1b2 | Dynamic beacon adjustment + exe lock fix | Callback chain + timeouts |
| fde7877 | 7 architecture safety fixes (locks, UUID, I/O→breaker) | +122 − 72 lines |
| e4ee3b9 | Remove beacon, add honest pagefile assessment | +42 − 303 lines |

All 7 existing tests pass across all commits. No regressions introduced.

# 8. Recommendation

Pursue Path B (low-speed compute expansion) as the primary differentiator, with Path A (smart pagefile on fixed USB) as a quick win for compatible devices. The combination provides:

- Immediate value for fixed USB SSD users (real Windows pagefile, zero overhead)

- Genuine technical moat via layer-aware prefetching on slow removable storage

- Honest reporting — no fake numbers, no cosmetic tricks, real measured performance

- Clear target market — resource-constrained devices where C:\ space is genuinely insufficient